| Direct Kinematics | Analysis |
| --- | --- |
|  | We observe that: x = L1 cos(θ1) + L2 cos(θ1 + θ2) y = L1 sin(θ1) + L2 sin(θ1 + θ2) which is the direct kinematics analysis for the robot for its two first articulations We set φ = θ1 + θ2 |
| Inversed Kinematics | Analysis |
|  | From Pythagora’s theorem: OB2=x2+ y2 Moreover OB2 = (L1 + L2 cos(θ2))2 + (L2 sin(θ2))2= L22sin(θ2)2+ L12+ L22cos(θ2)2+2 L1 L2 cos(θ2) ⟺ x2 + y2 - L12 - L22(cos(θ2)2+ sin(θ2)2) = 2 L1 L2 cos(θ2) |

So we take:

x2 + y2 - L12 - L22= 2 L1 L2 cos(θ2) ⟺ cos(θ2) =  x2 + y2 - L12 - L222 L1 L2

θ2 = arcos(x2 + y2 - L12 - L222 L1 L2)

Now for θ1:

φ = θ1 + θ2 ⟺ θ1 = φ - θ2 but what if we don’t know φ? Well, in that case:

ABO'triangle:
sin(θ2) = 1 - cos(θ2)2

ΒΟAcorner of  BOO'triangle:
ΒΟAcorner =arctan( BO'L1 + AO')= arctan( L2sin(θ2)L1 + L2cos(θ2))

Let’s say that ΒΟAcorner + θ1 = γ so:

θ1 = γ - ΒΟAcorner = arctan(yx) - arctan( L2sin(θ2)L1 + L2cos(θ2))